Принцип отображения флага в строке RDS на экране 4 дюйма.

В файле flag.ini параметры записываются в строку следующего формата:

LNG, X, Y, R, COLOR

LNG (String)

X, Y (int)

R, COLOR (unsigned int)

Флаг формируется из строк, в которых параметр LNG такой же как в языковом файле в первой строке.

В каждой строке записаны параметры для отображения закрашенной цветом COLOR окружности с центром в точке X, Y и радиусом R. (X и Y могут быть отрицательными)

Окружности отображаются в порядке следования в файле flag.ini

Размер видимой области (прямоугольник синего цвета) a = 480, b = 25.

Начало координат по оси X в видимой области перемещается от 0 до 640 для обеспечения эффекта движения. Для правильного отображения эффекта появления флага в левой части экрана следует отображать все его элементы за пределами левого края видимой области.

Цвет окружности состоит из 16 бит и формируется по следующей схеме от старшего к младшему биту:

RED(5bit) - GREEN(6bit) - BLUE(5bit).

Последняя окружность должна быть наименьшего диаметра и черного цвета (COLOR = 0) для отсечения лишних элементов справа от флага.

The principle of displaying the flag in the RDS line on a 4-inch screen.

In the flag.ini file, the parameters are written to a string of the following format:

LNG, X, Y, R, COLOR

LNG (String)

X, Y (int)

R, COLOR (unsigned int)

The flag is formed from lines in which the LNG parameter is the same as in the language file in the first line.

Each line contains parameters for displaying a COLOR-filled circle centered at X, Y and radius R. (X and Y can be negative)

The circles are displayed in order in the flag.ini file

The size of the visible area (blue rectangle) is a = 480, b = 25.

The origin of the X-axis coordinates in the visible area moves from 0 to 640 to ensure the effect of movement. To correctly display the effect of the flag appearing on the left side of the screen, all its elements should be displayed outside the left edge of the visible area.

The color of the circle consists of 16 bits and is formed according to the following scheme from the highest to the lowest bit:

RED(5bit) - GREEN(6bit) - BLUE(5bit).

The last circle must be of the smallest diameter and black (COLOR = 0) to cut off unnecessary elements to the right of the flag.